Experiment. No.01

Title: Study of Electromagnetic Spectrum Allocation and Wireless Bands Used in Communication

Aim: Study of Electromagnetic Spectrum Allocation and Wireless Bands Used in Communication

Requirement:

  Desktop/Laptop

  Internet Connection

Theory :

## 1. Electromagnetic Spectrum

The electromagnetic spectrum is the complete range of electromagnetic waves arranged according to their frequency or wavelength. Electromagnetic waves travel through free space at the speed of light and do not require a physical medium.

The relationship between frequency and wavelength is

C  =   f / λ

Where

- c = Speed of Light = 3×1083\times10^83×108 m/s

- f = Frequency (Hz)

- λ = Wavelength (m)

### Important Facts

- High frequency → Short wavelength

- Low frequency → Long wavelength

- High-frequency signals carry more data but cover shorter distances.

- Low-frequency signals travel farther and penetrate obstacles better.

## 2. Electromagnetic Spectrum Regions

| Region |  | Frequency Range | Applications |
| --- | --- | --- | --- |
| Radio Waves | 3 kHz – 300 MHz |  | Radio, TV |
| Microwaves | 300 MHz – 300 GHz |  | Mobile, Wi-Fi, Radar |
| Infrared | 300 GHz – 430 THz |  | Remote Controls |
| Visible Light | 430 – 770 THz |  | Optical Fiber Communication |
| Ultraviolet | Above Visible Light |  | Sterilization |
| X-Rays | High Frequency |  | Medical Imaging |
| Gamma Rays | Highest Frequency |  | Cancer Therapy |

## 3. Radio Frequency Bands

Wireless communication mainly uses radio and microwave frequencies.

| Band | Frequency Range | Common Applications |
| --- | --- | --- |
| VLF | 3–30 kHz | Navigation |
| LF | 30–300 kHz | Long-distance Communication |
| MF | 300 kHz–3 MHz | AM Radio |
| HF | 3–30 MHz | Shortwave Radio |
| VHF | 30–300 MHz | FM Radio, Television |
| UHF | 300 MHz–3 GHz | Mobile Communication |
| SHF | 3–30 GHz | Wi-Fi, Radar, Satellite |
| EHF | 30–300 GHz | 5G mmWave |

## 4. Spectrum Allocation

Spectrum allocation is the process of assigning different frequency bands to different communication services to prevent interference and ensure efficient use of the radio spectrum.

### Regulatory Organizations

### International

- International Telecommunication Union (ITU)

### India

- Department of Telecommunications (DoT)

- Wireless Planning & Coordination (WPC) Wing

## 5. Licensed and Unlicensed Spectrum

### Licensed Spectrum: Frequency bands that are assigned exclusively to specific operators through government auctions. Cellular networks (2G, 3G, 4G, 5G) operate in licensed spectrum to guarantee Quality of Service (QoS) and controlled interference.

### Unlicensed Spectrum: Frequency bands that can be used without a license, provided devices comply with power limits. The ISM bands (2.4 GHz, 5 GHz) are the most prominent examples, used by Wi-Fi, Bluetooth, and Zigbee.

### Licensed Spectrum

Requires government permission.

Examples:

- Mobile Networks

- TV Broadcasting

- Satellite Communication

### Unlicensed Spectrum

Can be used without obtaining a license (within specified limits).

Examples:

- Wi-Fi

- Bluetooth

- ZigBee

## 6. Common Wireless Communication Bands

| Technology | Operating Frequency | Application |
| --- | --- | --- |
| AM Radio | 535–1705 kHz | Radio Broadcasting |
| FM Radio | 88–108 MHz | Music Broadcasting |
| Television | 54–806 MHz | TV Transmission |
| GSM (2G) | 900 MHz, 1800 MHz | Voice Calls |
| 3G | 2100 MHz | Mobile Internet |
| 4G LTE | 700 MHz, 1800 MHz, 2300 MHz | High-Speed Data |
| 5G NR | 700 MHz, 3.5 GHz, 26 GHz | High-Speed Communication |
| Wi-Fi | 2.4 GHz, 5 GHz, 6 GHz | Wireless Internet |
| Bluetooth | 2.4 GHz | Personal Area Network |
| GPS | 1575.42 MHz | Navigation |
| RFID | 125 kHz, 13.56 MHz, 900 MHz | Identification |
| NFC | 13.56 MHz | Contactless Payment |
| Satellite | L, S, C, Ku, Ka Bands | Satellite Communication |

# Applications

- Mobile Communication

- Wi-Fi Networks

- Bluetooth Communication

- Satellite Communication

- GPS Navigation

- Radio Broadcasting

- Television Broadcasting

- Internet of Things (IoT)

- Smart Homes

- Aviation Communication

- Defense Communication

# Advantages

- Efficient utilization of available spectrum.

- Reduced interference.

- Reliable communication.

- Supports multiple wireless technologies.

- Enables high-speed wireless communication.

Conclusion: The electromagnetic spectrum and wireless communication frequency bands were successfully studied. The operating frequencies of common wireless technologies such as mobile communication, Wi-Fi, Bluetooth, GPS, and FM radio were identified, and the concepts of spectrum allocation, licensed spectrum, and unlicensed spectrum are understood.