Experiment. No.02

Title: Simulation of Basic Communication System (Tx–Channel–Rx) Using Python

Aim: Simulation of Basic Communication System (Tx–Channel–Rx) Using Python

Requirement:

  Python 3.x

  NumPy

  Matplotlib

Theory :

A communication system transfers information from a source to a destination.

It consists of three basic blocks:

### a) Transmitter (Tx)

The transmitter generates and prepares the signal for transmission.

Functions:

- Signal generation

- Encoding

- Modulation (not included in this basic experiment)

### (b) Communication Channel

The communication channel carries the transmitted signal.

Examples:

- Air

- Optical Fiber

- Coaxial Cable

- Satellite Link

During transmission, noise is added to the signal.

### (c) Receiver (Rx)

The receiver receives the noisy signal and recovers the original information.

Functions:

- Signal reception

- Filtering

- Demodulation (not included here)

# Algorithm

- Import the required Python libraries.

- Generate a time axis.

- Generate a sine wave as the message signal.

- Generate random Gaussian noise.

- Add the noise to the message signal.

- Assume the receiver receives the noisy signal.

- Plot:

  - Original signal

  - Noise

  - Received signal

## Procedure:

1.Go to https://colab.research.google.com

2.Click New Notebook.

3.Copy the Python code  provided.

4.Paste it into the first code cell.

5.Click the ▶ Run button or press Shift + Enter.

6.Introduce low and high AWGN noise in the code and observe the effect of noise on the received bits.

Python Code

For Analog signal

# Simulation of Basic Communication System (Tx-Channel-Rx)

import numpy as np

import matplotlib.pyplot as plt

# -------------------------

# Transmitter

# -------------------------

# Time axis

t = np.linspace(0, 1, 1000)

# Original message signal

frequency = 5       # Hz

message = np.sin(2 * np.pi * frequency * t)

# -------------------------

# Communication Channel

# -------------------------

# Generate Gaussian Noise

noise = np.random.normal(0, 0.3, len(t))

# Signal after passing through channel

received_signal = message + noise

# -------------------------

# Receiver

# -------------------------

# In this simple simulation, receiver directly receives noisy signal

recovered_signal = received_signal

# -------------------------

# Plotting

# -------------------------

plt.figure(figsize=(12,8))

plt.subplot(3,1,1)

plt.plot(t, message, color='blue')

plt.title("Original Message Signal")

plt.xlabel("Time (s)")

plt.ylabel("Amplitude")

plt.grid(True)

plt.subplot(3,1,2)

plt.plot(t, noise, color='red')

plt.title("Noise Added by Channel")

plt.xlabel("Time (s)")

plt.ylabel("Noise")

plt.grid(True)

plt.subplot(3,1,3)

plt.plot(t, recovered_signal, color='green')

plt.title("Received Signal")

plt.xlabel("Time (s)")

plt.ylabel("Amplitude")

plt.grid(True)

plt.tight_layout()

plt.show()

# For digital signal

## 1. Transmitter (Tx)

Functions:

- Generate binary information

- Encode data

- Modulate (BPSK)

Example:

Input bits

10110010

↓

BPSK symbols

+1 -1 +1 +1 -1 -1 +1 -1

## 2. Channel

The channel introduces impairments such as:

- Additive White Gaussian Noise (AWGN)

- Fading

- Interference

Simulation:

Original

+1 -1 +1

↓

Noise Added

0.82 -0.73 1.15

## 3. Receiver (Rx)

Functions:

- Detect received symbols

- Demodulate

- Recover transmitted bits

Decision rule:

If signal > 0 → bit = 1

If signal < 0 → bit = 0

# Typical Output

Transmitted:

[1 0 1 1 0 1 0 0]

Received:

[1 0 1 1 0 1 0 0]

With lower Signal-to-Noise Ratio (SNR), some bits may be received incorrectly due to increased noise.

# Expected Results

- At high SNR (15–20 dB):

  - Very few or no bit errors.

  - Received signal closely matches the transmitted signal.

- At moderate SNR (5–10 dB):

  - Some noise is visible.

  - Most bits are still correctly recovered.

- At low SNR (0–5 dB):

  - Noise significantly distorts the signal.

  - Higher probability of bit errors.

Python Code

import numpy as np

import matplotlib.pyplot as plt

# Generate binary data

N = 20

tx = np.random.randint(0, 2, N)

# BPSK Modulation

signal = 2*tx - 1

# Add Gaussian Noise

noise = np.random.normal(0, 0.4, N)

received = signal + noise

# Detection

rx = (received > 0).astype(int)

print("Transmitted:", tx)

print("Received:", rx)

plt.figure(figsize=(8,6))

plt.subplot(3,1,1)

plt.stem(tx)

plt.title("Transmitted Bits")

plt.subplot(3,1,2)

plt.plot(received,'o-')

plt.grid(True)

plt.title("Signal after Channel")

plt.subplot(3,1,3)

plt.stem(rx)

plt.title("Recovered Bits")

plt.tight_layout()

plt.show()

# Output

# Applications

- Mobile communication

- Wi-Fi

- Bluetooth

- Satellite communication

- Optical communication

- IoT systems

- Radar systems

# Advantages

- Easy to understand.

- Demonstrates the Tx–Channel–Rx model.

- No additional hardware required.

- Useful for beginners in communication engineering.

- Can be extended for digital communication experiments.

Conclusion:

1.The basic communication system was successfully simulated using Python. The transmitter generated a sine wave, the communication channel added Gaussian noise, and the receiver obtained the noisy signal. The simulation demonstrated the effect of channel noise on the quality of the received signal.

2.For digital signal

This simulation demonstrates the basic operation of a digital communication system comprising a Transmitter (Tx), a Channel, and a Receiver (Rx).

Random binary data are modulated using Binary Phase Shift Keying (BPSK), transmitted through an AWGN channel, and then demodulated at the receiver.

As the channel noise increases (lower SNR), the received signal becomes more distorted, leading to a higher likelihood of bit errors. This simple model forms the foundation for understanding more advanced digital communication systems used in modern wireless technologies.