Experiment No.07

Title: Data transfer using Bluetooth and analysis of performance.

# EXPERIMENT NO. 07

Aim: To transfer data between two Bluetooth-enabled mobile devices and analyze the performance of Bluetooth communication based on transfer time, data size, and data transfer rate.

Requirement:

## Hardware

  Two Bluetooth-enabled smartphones

  Bluetooth-enabled laptop/PC (optional)

  Measuring tape

  Open area or laboratory space

Software

  Bluetooth facility available on Android/iOS devices

  A Bluetooth file-sharing or communication application, if required

  File manager/gallery for transferring a test file

Theory :

## What is Bluetooth?

Bluetooth is a short-range wireless communication technology used to exchange data between electronic devices without using physical cables.

It operates primarily in the 2.4 GHz ISM (Industrial, Scientific and Medical) band.

Bluetooth is commonly used for:

- Wireless headphones

- Speakers

- Smartwatches

- Keyboards and mice

- Smartphones

- Fitness trackers

- IoT devices

- File and data exchange

## Basic Bluetooth Communication

A typical Bluetooth connection involves two devices:

Device A                         Device B

┌────────────┐                   ┌────────────┐

│ Smartphone │  <--- Bluetooth ->│ Smartphone │

└────────────┘                   └────────────┘

│                              │

└──── Wireless 2.4 GHz ───────┘

Before communication can take place, the devices generally need to:

- Enable Bluetooth.

- Discover each other.

- Select the required device.

- Perform pairing/authentication.

- Establish a connection.

- Exchange data.

# Performance Parameters

Bluetooth performance can be analyzed using several parameters.

## Transfer Time

Transfer time is the time required to send a file from the sender to the receiver.

It is measured in:seconds (s)

## Data Transfer Rate

The effective data transfer rate can be calculated as:

Transfer Rate= Data Size​​ / Transfer Time

If file size is expressed in megabytes:

Rate (MB/s)= File Size (MB)​​ / Time (s)

For Mbps:

Rate (Mbps)= File Size (MB)×8​​ / Time (s)

## Example:

Suppose a 20 MB file takes 10 seconds to transfer.

Rate=1020​ =2 MB/s

Converting to Mbps:

2×8=16 Mbps

Therefore:

Effective transfer rate = 16 Mbps

# Procedure

## Part A — Pairing

- Switch ON Bluetooth on both smartphones.

- Open Bluetooth settings.

- Search for available devices.

- Select the second smartphone.

- Confirm the pairing request/passkey.

- Verify that both devices are paired.

## Part B — File Transfer

- Select a test file on the sender smartphone.

- Select Share → Bluetooth or the equivalent Bluetooth-sharing option.

- Select the paired receiver.

- Accept the transfer on the receiving device.

- Start the stopwatch when the transfer begins.

- Stop the stopwatch when the transfer completes.

- Record the transfer time.

- Verify that the received file can be opened successfully.

Reference Observation Table — File Size vs Transfer Time

| Sr. No. | File Type | File Size (MB) | Transfer Time (s) | Transfer Rate (MB/s) | Transfer Rate (Mbps) |
| --- | --- | --- | --- | --- | --- |
| 1 | Text | 1 | 1.2 | 0.83 | 6.67 |
| 2 | Image | 5 | 3.5 | 1.43 | 11.43 |
| 3 | Audio | 10 | 7.0 | 1.43 | 11.43 |
| 4 | Video | 25 | 18.0 | 1.39 | 11.11 |
| 5 | Large Video | 50 | 37.0 | 1.35 | 10.81 |

# Program

# import matplotlib.pyplot as plt

# # Experimental data

# file_size = [1, 5, 10, 25, 50]       # MB

# transfer_time = [1.2, 3.5, 7.0, 18.0, 37.0]  # seconds

# # Calculate transfer rate

# transfer_rate = []

# for size, time in zip(file_size, transfer_time):

# rate = (size * 8) / time

# transfer_rate.append(rate)

# # Display results

# print("File Size (MB)   Time (s)   Rate (Mbps)")

# for size, time, rate in zip(file_size, transfer_time, transfer_rate):

# print(f"{size:10}      {time:6.2f}      {rate:8.2f}")

# # Plot

# plt.figure(figsize=(8, 5))

# plt.plot(

# file_size,

# transfer_rate,

# marker='o'

# )

# plt.xlabel("File Size (MB)")

# plt.ylabel("Transfer Rate (Mbps)")

# plt.title("Bluetooth Data Transfer Performance")

# plt.grid(True)

# plt.show()

# Expected Output

The Python program displays a table similar to:

File Size (MB)   Time (s)   Rate (Mbps)

1          1.20          6.67

5          3.50         11.43

10          7.00         11.43

25         18.00         11.11

50         37.00         10.81

A graph showing transfer rate versus file size is also generated.

# Output

# Advantages

- Wireless communication

- Low power consumption

- Simple pairing

- Low-cost implementation

- Suitable for short-range communication

- Supports many types of peripherals

- Does not require a wired connection

# Limitations

- Limited communication range

- Lower throughput than many Wi-Fi connections

- Performance can be affected by obstacles

- 2.4 GHz interference may affect communication

- Device compatibility can vary

- Actual range depends strongly on the implementation

Conclusion:

Bluetooth is suitable for short-range wireless communication and low-to-moderate data-transfer applications. Performance analysis using actual transfer measurements provides a better understanding of the difference between theoretical Bluetooth capabilities and practical throughput.