Experiment No.08

Title: Analysis of multipath effects (signal fluctuation observation).

Aim: To observe and analyze the effect of multipath propagation on received wireless signal strength by measuring signal fluctuations at different receiver positions.

Requirement:

## Hardware

 Wi-Fi router/access point

 Laptop with Wi-Fi adapter

 Measuring tape

 Open laboratory space

 Obstacles/walls in the environment

Software

 Python 3.x

 Matplotlib

 NumPy

Theory :

## Multipath Propagation

In wireless communication, a transmitted radio signal may reach the receiver through multiple paths.

This occurs because radio waves can undergo:

- Reflection

- Diffraction

- Scattering

As a result, the receiver may receive several copies of the same transmitted signal at slightly different times.

The receiver therefore receives the signal through different paths.

# Multipath Signal

The received signal can be considered as the combination of multiple signal components:

r(t)=i=1∑n​Ai​s(t−τi​)

where:

- Ai​ = amplitude of the signal arriving through path i

- τi​ = delay associated with path i

- s(t) = transmitted signal

- r(t) = received signal

Because the different signal components can have different phases, they may either reinforce or cancel one another.

# Multipath Fading

The rapid variation in received signal strength caused by multipath propagation is called multipath fading.

A receiver may therefore observe:

Position →     1   2   3   4   5   6   7   8

Signal →      -45 -48 -42 -55 -47 -63 -50 -44

Even though the transmitter has not changed its power, the measured signal can fluctuate considerably.

# Causes of Multipath

Multipath is commonly caused by:

- Buildings

- Walls

- Furniture

- Vehicles

- Metal objects

# Procedure

### Step 1 — Setup

- Place the Wi-Fi router at a fixed location.

- Connect the laptop to the router.

- Keep the router stationary throughout the experiment.

- Select a measurement path in the laboratory.

- Mark positions at regular intervals, for example:

  - 1 m

  - 2 m

  - 4 m

  - 6 m

  - 8 m

  - 10 m

### Step 2 — RSSI Measurement

- Place the laptop at the first position.

- Measure the Wi-Fi signal strength.

- Record the value.

- Take at least three readings at each position.

- Calculate the average RSSI.

- Move the laptop to the next position.

- Repeat the measurement.

- Continue until all positions have been measured.

# Measuring Wi-Fi Signal on Windows

Open Command Prompt and enter:

netsh wlan show interfaces

The output may contain:

Signal : 82%

Windows normally reports the Wi-Fi signal as a percentage.

For this experiment, a Wi-Fi analysis tool that directly provides signal strength in dBm is preferable if available.

# Program

# import matplotlib.pyplot as plt

# import numpy as np

# # Distance from router in metres

# distance = np.array([1, 2, 3, 4, 5, 6, 7, 8])

# # Average RSSI obtained from experiment

# rssi = np.array([-45, -49, -43, -55, -47, -62, -51, -46])

# plt.figure(figsize=(9, 5))

# plt.plot(

# distance,

# rssi,

# marker='o',

# linewidth=2

# )

# plt.xlabel("Distance from Router (m)")

# plt.ylabel("Average RSSI (dBm)")

# plt.title("Analysis of Multipath Effects")

# plt.grid(True)

# # More negative RSSI = weaker signal

# plt.gca().invert_yaxis()

# plt.show()

# Expected Output

The graph should show fluctuations in received signal strength.

RSSI (dBm)

-40 |       ●

-45 | ●           ●           ●

-50 |    ●              ●

-55 |          ●

-60 |                ●

|

+------------------------------

1  2  3  4  5  6  7  8

Distance (m)

The important feature is that the RSSI does not necessarily follow a smooth decreasing curve.

# Output

# Applications

Multipath analysis is useful in:

- Wi-Fi network planning

- 4G/LTE networks

- 5G networks

- Wireless Sensor Networks

- IoT networks

- Indoor positioning

- Mobile communication

Conclusion: Multipath-induced fluctuations in received wireless signal strength were successfully observed. The experiment demonstrated the effects of constructive and destructive interference on the received Wi-Fi signal.