Experiment. No.09

Title: Study of cellular network parameters using smartphones (signal strength, network type 2G/3G/4G/5G).

Aim: To study and measure important cellular network parameters using a smartphone and identify the network technology being used, such as 2G, 3G, 4G, and 5G.

Requirement:

  Desktop/Laptop

  Smartphone

  Internet Connection

Theory :

A cellular network divides a geographical area into smaller regions called cells. Each cell is served by a cellular base station.

A smartphone communicates with the cellular network through the nearest suitable cell site.

The quality of a cellular connection depends on several factors, including:

- Distance from the cell tower

- Obstacles such as buildings

- Indoor/outdoor location

- Frequency band

- Network technology

- Interference

- Number of users

- Terrain

# Cellular Network Generations

| Generation | Technology | Typical Characteristics |
| --- | --- | --- |
| 2G | GSM | Voice, SMS, basic data |
| 3G | UMTS/HSPA | Voice and higher-speed mobile data |
| 4G | LTE | High-speed IP-based communication |
| 5G | 5G NR | Higher capacity, lower latency and higher data rates |

The exact network type displayed by a smartphone depends on the handset, SIM, operator, coverage and current network conditions.

# Important Cellular Parameters

## 1 Network Type

The network type indicates the cellular technology to which the phone is currently connected.

Examples:

GSM       → 2G

EDGE      → 2G

UMTS      → 3G

HSPA      → 3G

LTE       → 4G

NR        → 5G

A smartphone may switch automatically between technologies depending on availability and network conditions.

## 2 Signal Strength

Signal strength indicates how strongly the smartphone receives the cellular signal.

It is commonly represented in dBm.

The value is generally negative:

-60 dBm

-80 dBm

-100 dBm

-110 dBm

For received power, a value closer to zero generally represents a stronger signal.

For example:

-60 dBm  → stronger

-80 dBm  → weaker

-100 dBm → much weaker

Important: Signal strength alone does not completely determine user-perceived network performance. Interference, congestion, signal quality and network configuration can also affect performance.

## 3 Signal Quality

Depending on the network technology, smartphones may display additional measurements such as:

- RSRP

- RSRQ

- SINR/SNR

- RSSI

These parameters are particularly useful when studying LTE/4G and 5G networks.

### RSRP

Reference Signal Received Power (RSRP) represents the received power of specific reference signals and is commonly used for LTE/5G radio measurements.

### RSRQ

Reference Signal Received Quality (RSRQ) provides information about received signal quality and interference/loading conditions.

### SINR

Signal-to-Interference-plus-Noise Ratio (SINR) indicates how strong the desired signal is relative to interference and noise.

Generally, higher SINR indicates better radio conditions.

Conclusion: The cellular network parameters were successfully studied using a smartphone.